MII PACKAGING MANAGEMENT

Standard Operating Procedure

Packaging Procurement · Approval Workflow · Sample Validation · Testing · Production Release

| Document Number | MII-PKG-SOP-001 |
| --- | --- |
| Version | 1.0 (March 2026) |
| Process | Unified end-to-end packaging management — Initiation through production release |
| Process Owner | MII(Mukesh/Satyam) |
| Approvers | Quality Manager (QM), Brand Manager (BM) & Product Manager |

Table of Contents

1.  Purpose

2.  Scope

3.  Roles & Responsibilities

4.  Definitions & Abbreviations

5.  Packaging Tracker — Overview & Column Reference

6.  End-to-End Process Flow (Phases P1 – P11)

7.  Packaging Specification Standards

8.  Artwork Requirements & BM Approval

9.  Sample Preparation Requirements

10.  Drop Test Procedure (ISTA 1A / 2A / 6-Amazon)

10.1 Test Conditions & Environment

10.2 Drop Sequence

10.3 Acceptance Criteria

10.4 Rejection Criteria

10.5 Post-Test Evaluation Checklist

10.6 Iteration Process on Failure

10.7 Specification & Artwork Freeze

11.  Approval Matrix (RACI)

12.  Quality Checkpoints

13.  Timelines & Milestone Management

14.  Tracker Maintenance & Data Integrity

15.  Escalation Matrix

16.  Key Suppliers & Document Control

1.  Purpose

This Standard Operating Procedure defines the unified, end-to-end process for managing MII product packaging — from the initiation of a packaging requirement through artwork development, specification approval, sample validation, drop test certification, functional testing, production release, incoming inspection, and final closure. It brings together procurement governance and quality validation into a single process, ensuring every packaging item is fully tested and approved before bulk production is authorised.

2.  Scope

This SOP applies to all MII product lines and packaging components listed below:

- Fans — Lit, Folde, Halo, BFF series

- Vacuum Cleaners — Ren Pro, Ren Go, Rengo

- Kitchen appliances — Brisk, Krush, Electric Kettle series

- Grooming & Garment Care — Multi Styler, Storm Go

- Spare Parts & Accessories

This SOP applies in full for all of the following scenarios:

- All new product, spare, and renew packaging developments

- Packaging revisions — material, dimension, or structural artwork changes

- Vendor or supplier changes

- Cost reduction projects impacting packaging structure

- Re-validation during mass production where required

3.  Roles & Responsibilities

| Role / Function | Responsibility |
| --- | --- |
| Project Lead (Mukesh / Satyam) | Initiates packaging requirement; coordinates artwork and spec; drives supplier engagement; manages tracker; ensures timely delivery |
| Brand Manager (BM) | Reviews and approves packaging artwork for brand compliance, colour accuracy, and print-readiness |
| Quality Manager (QM) | Reviews and approves packaging specifications; leads drop test oversight; raises NCRs; signs off sample approval |
| PM (Product Manager) | Review and approve the artwork content & provide final approval before mass production |
| Supplier | Manufactures packaging; submits samples; conducts modifications on failure; provides test samples per frozen spec |
| Partner | Receives and inspects incoming bulk materials; updates available quantity in tracker |

4.  Definitions & Abbreviations

| Term | Definition |
| --- | --- |
| SOP | Standard Operating Procedure |
| Artwork | Final print-ready design file for packaging (box or sticker) |
| Specification (Spec) | Technical parameters — ply count, GSM, dimensions, material, lamination |
| UOM | Unit of Measure (Nos. = Numbers / pieces) |
| GSM | Grams per Square Metre — measure of paper or board density |
| PLY | Number of corrugated board layers in a box |
| FTS Sticker | Fit to sale sticker for spare / service parts |
| SKU | Stock Keeping Unit — unique identifier per product variant |
| NCR | Non-Conformance Report — formal record of a quality rejection |
| Spec Freeze | Formal lock of packaging specification — no changes permitted post-freeze without revalidation and ESN |

5.  Packaging Tracker — Overview & Column Reference

The MII Packaging Tracker is the single source of truth for all active packaging orders. It contains three sheets:

| Sheet | Purpose | Key Users |
| --- | --- | --- |
| MII Packaging Tracker | Tracks all active and completed packaging items with full details — artwork, specs, qty, supplier, approval, drop test status, and remarks | Project Leads, QM, BM, PM & Suppliers |
| MII Upcoming Project List | Captures future packaging requirements across all product categories | PM, BM, Project Leads |
| Timelines | Quarterly 2026 launch roadmap by product and category | PM, Leadership, Project Leads |

5.1 Main Tracker Column Reference

| Col | Field | Description |
| --- | --- | --- |
| A | Sr. No. | Sequential item number |
| B | Product | Product name (e.g., Ren Pro, Halo v2, Brisk) |
| C | Item Description | Specific packaging component (e.g., Outer Box Black, Product Sticker Grey) |
| D | Purpose | Intended use (e.g., For renew desk, For Brisk production) |
| E | Artwork | Artwork receipt status: Received / Not Received |
| F | Specification | Technical parameters — Ply, GSM, size, material type,BS. |
| G | Qty | Required quantity to be procured |
| H | Available Qty | Current on-hand stock — actual physical count only |
| I | UOM | Unit of Measure (typically Nos.) |
| J | QM & BM Approval | Sign-off status: Approved / Pending — set only after both approve |
| K | Supplier Name | Vendor responsible for supply |
| L | Status | Order status: Open (in progress) / Close (fulfilled) |
| M | Responsibility | Project Lead assigned to this item |
| N | Target Date | Expected delivery date |
| O | Final Approved Artwork | Confirmation that artwork is locked and shared with supplier |

Tracker Link :

https://docs.google.com/spreadsheets/d/1K7xBskgUoNoGEXwjRVQHEJb1HF8cqrku/edit?usp=sharing&ouid=100628659654802156466&rtpof=true&sd=true

6.  End-to-End Process Flow

The MII packaging process runs through eleven sequential phases. No phase may be skipped. The testing (Phases P5–P8) is mandatory pre-production gate — bulk production cannot begin until a PASS verdict is achieved and the specification is formally frozen.

| P1 PHASE | Requirement Initiation Owner: Project Lead |
| --- | --- |
|  | → Add new row to MII Packaging Tracker; assign Sr. No., Product, Item Description, Purpose → Enter required Qty, UOM, and Target Date (aligned to production schedule) → For future launches, add to MII Upcoming Project List first |
| GATE CONDITION | Production plan or launch schedule confirmed; Target Date set |

| P2 PHASE | Artwork Submission & Brand Manager Review Owner: Project Lead + Brand Manager (BM) |
| --- | --- |
|  | → Request artwork files from Design / Brand team → Artwork must be print-ready: CMYK, min. 300 DPI, 3mm bleed, correct dyeline → BM reviews for brand compliance, colour accuracy, typography, regulatory text → If rejected: Design team revises; allow 3–5 days per revision round → SLA: BM review within 3 working days of artwork receipt |
| GATE CONDITION | BM has reviewed and confirmed artwork is print-ready and brand-compliant |

| P3 PHASE | Specification Finalisation & QM Review Owner: Project Lead + Quality Manager (QM) + Product Manager(PM) |
| --- | --- |
|  | → Define all technical parameters — Box Type, PLY count, GSM, Flute type,B.S dimensions (L×W×H mm) → For stickers: Paper type, size (W×H mm), lamination,finish, adhesive strength → Verify dimensions against actual product for fit before submitting to QM → QM reviews spec for quality compliance and structural suitability → SLA: QM review within 3 working days of spec submission |
| GATE CONDITION | QM has signed off on the specification; dimensions verified against product |

| P4 PHASE | Joint QM + BM Final Approval Owner: Project Lead (facilitates) + QM + BM +PM |
| --- | --- |
|  | → Share finalised artwork and approved specification simultaneously to both QM and BM → QM verifies spec on the artwork dieline — dimensions, structural marks, regulatory zones → BM verifies artwork on the approved dieline — brand elements, copy, colour → Both must approve independently — one approval cannot substitute the other → On rejection: Log feedback in Remarks; return to Phase 2 or 3 as applicable → SLA: Joint approval within 3 working days of concurrent submission |
| GATE CONDITION | Both QM and BM have formally approved the artwork + specification combination |

SAMPLE APPROVAL BEFORE PRODUCTION — PHASES P5 THROUGH P8

|  | Phases P5 through P8 form the mandatory sample approval gate. No purchase order for bulk production may be placed, and no production run may begin, until all samples have achieved a PASS verdict in the drop test and the specification has been formally frozen (P8). This is a non-negotiable requirement. |
| --- | --- |

| P5 PHASE | Sample Preparation & Supplier Submission Owner: Project Lead + Supplier |
| --- | --- |
|  | → Instruct supplier to prepare minimum 3 fully packed finished goods (5 for high-risk products) → Each sample MUST include: final shipper carton + inner box (if applicable) + approved cushioning material (EPE / Thermocol / Moulded pulp) + actual fully assembled product inside → Samples must represent the exact production-intent configuration → Hand-cut samples are permitted for initial validation only — GSM and paper spec must be identical to approved spec → Final approval requires production-line samples exclusively — no hand-cut allowed at this gate → Supplier submits samples to QM team with sample tag and version number → QM team confirms sample receipt and logs in tracker Remarks |
| GATE CONDITION | Min. 3 production-intent samples received by QM team and tagged with version number |

| P6 PHASE | Testing Owner: QM Team + Product Team (observed by MII) |
| --- | --- |
|  | → Drop surface: Solid concrete floor or steel impact plate — flat, level, non-cushioned, debris-free → Execute functional testing as per Nuuk standard. → Video record the entire test; photograph each sample before and after → Do not open samples between drops — inspect only after full sequence is complete |
| GATE CONDITION | All drop orientations completed per minimum req of ISTA standard; video and photo records captured |

| P7 PHASE | Post-Drop Evaluation & PASS / FAIL Verdict Owner: QM Team + Product Team + MII |
| --- | --- |
|  | → E1: 360° visual inspection of carton exterior — photograph all faces → E2: Open carton; inspect all internal faces and insert panels → E3: Inspect cushioning alignment — measure and record any displacement (mm) → E4: Inspect product surface — check for cracks, chips, scratches, housing deformation → E5: Perform full functional test — power-on, operate all product functions per test plan → E6: Complete signed test checklist; annotate all observations on photographs → E7: Map all findings against acceptance and rejection criteria (see Section 10.3–10.4) → Issue PASS or FAIL verdict with written justification → If FAIL: Initiate iteration process (Phase 7a — see Section 10.6) |
| GATE CONDITION | Written PASS / FAIL verdict issued by QM; signed test checklist, photos, and video filed |

| P7a IF FAIL | Iteration on Failure — Loop back to P5 after modification Owner: Supplier (modifies) + QM Team (re-tests) + Project Lead (tracks) |
| --- | --- |
|  | 1. Conduct Root Cause Analysis (RCA) — identify the exact failure mode and packaging component 2. Modify as required — permissible modifications: • Increase cushion thickness (EPE / foam) • Upgrade flute type: 3-ply → 5-ply → 7-ply • Increase liner paper GSM per QM recommendation • Revise fitment accuracy or internal partitioning • Add corner inserts or edge protection 3. Build revised sample — assign a new version number 4. Return to Phase P5 — re-submit revised sample to QM for new drop test cycle 5. Every iteration must be documented in the iteration log — no undocumented changes permitted |
| LOOP UNTIL PASS | Repeat Phases P5 → P6 → P7 until all samples achieve a PASS verdict. Each iteration loop must produce a new test report. |

| P8 PHASE | Specification & Artwork Freeze (Post-PASS) Owner: QM Manager + BM + Project Lead |
| --- | --- |
|  | → F1: Specification Freeze — issue frozen spec sheet; no further structural changes permitted → F2: Artwork Freeze — confirm final artwork matches approved dyeline and spec; assign version number and lock file → F3: Issue Controlled Version Document with QM sign-off; distribute to supplier, PM, and QM → F4: Share frozen spec and artwork with supplier in writing — supplier provides written confirmation of receipt → Update tracker Column O (Final Approved Artwork) and add remark: 'Spec frozen post drop test PASS — [date]' → No bulk order may be placed until this phase is complete |
| GATE CONDITION | Frozen spec sheet issued; artwork file locked; supplier written confirmation received |

| P9 PHASE | Supplier Award & Purchase Order Release Owner: Project Lead + Procurement + PM |
| --- | --- |
|  | → Confirm supplier based on approved sample and frozen spec → For boxes: Dhawan Packaging (standard); For stickers / print: C.V Print → Share frozen spec and approved artwork with selected supplier → Obtain supplier confirmation of lead time and pricing → PM approves Purchase Order; Project Lead places PO → Update tracker: Supplier Name (Col K) and note PO date in Remarks → No verbal release — all authorisations must be in writing |
| GATE CONDITION | Signed PO placed with supplier; lead time confirmed in writing |

| P10 PHASE | Production Monitoring & Incoming Material Inspection Owner: Project Lead + E-Pack / Warehouse + QM |
| --- | --- |
|  | → Track supplier production progress weekly against Target Date → Escalate immediately if delay >3 working days beyond agreed schedule → On receipt of bulk material: → Warehouse team performs 100% quantity count against PO → QM inspects: dimensions (±2mm), ply count, GSM, print registration, sticker adhesion → Photograph all faces of sampled cartons → If PASS: Update Available Qty (Col H); add dated remark 'Material received and inspected — [date]' → If FAIL: Place on hold; raise NCR; notify QM and PM same day; do not update Status |
| GATE CONDITION | Incoming inspection pass confirmed by QM; Available Qty updated in tracker |

| P11 PHASE | Closure & Record Archival Owner: Project Lead |
| --- | --- |
|  | → Confirm physical receipt of all materials and QM inspection clearance → Set Status (Col L) = 'Close' → Confirm Column O (Final Approved Artwork) is completed → Add closure remark with date: 'Closed — material received and accepted — [date]' → File all test records: drop test video, photos, signed checklist, iteration log (if any) → For cancelled items: Do NOT delete row — add remark 'CANCELLED — [reason]' and set Status = Close |
| GATE CONDITION | Status = Close; all records filed; no outstanding NCRs |

7.  Packaging Specification Standards

7.1 Corrugated Box Parameters

| Parameter | Description | Standard / Typical Values |
| --- | --- | --- |
| Box Type | Overall box configuration | Universal Brown Box · White Box · Custom Printed |
| No. of Ply | Corrugated board layers | 3-ply (standard) · 5-ply (heavy items) · 7-ply (high-risk) |
| GSM | Liner paper grammage | 150–200 GSM standard · 300+ GSM premium |
| Flute Type | Corrugated flute profile | B-flute · C-flute · E-flute (small boxes) |
| Dimensions (L×W×H) | Internal box dimensions | Verified against product ±2mm; confirmed pre-freeze |
| Moisture Resistance | Liner protection | Moisture-resistant liner mandatory for electronics packaging |
| Print Registration | Print alignment tolerance | ±1mm maximum misalignment |

7.2 Sticker & Label Parameters

| Parameter | Description | Standard / Typical Values |
| --- | --- | --- |
| Paper Type | Sticker substrate material | Chromo paper · Transparent film · PP / PE |
| Size (W×H) | Sticker dimensions | Confirmed against dieline |
| Lamination | Surface finish | Matt · Glossy · None |
| Adhesive Strength | Bond classification | Permanent · Removable · High-tack |
| Print Registration | Alignment tolerance | ±0.5mm maximum |
| Print Colours | Colour specification | 4C CMYK · Pantone spot colours |

8.  Artwork Requirements & BM Approval

All of the following must be confirmed before artwork is submitted for BM review. Non-compliant files will be rejected and returned to the Design team.

| # | Requirement | Standard | Mandatory? |
| --- | --- | --- | --- |
| A1 | File format | AI / PDF — print-ready with embedded fonts and linked images | YES |
| A2 | Colour mode | CMYK only — no RGB or unresolved Pantone mismatches | YES |
| A3 | Resolution | Minimum 300 DPI at actual print size | YES |
| A4 | Bleed | 1mm bleed on all sides | YES |
| A5 | Safe zone | 5mm safe zone from trim line for all critical text and logos | YES |
| A6 | Dieline accuracy | Confirmed against supplier-approved dieline and actual box / sticker dimensions | YES |
| A7 | Regulatory text | All mandatory warnings, certifications (BIS/ISI), MRP, MFD, and batch fields present | YES |
| A8 | Brand compliance | Logo placement, colour hex codes, and typography match current brand guide | YES |
| A9 | Copy proofread | All text verified — correct product name, SKU, no spelling errors | YES |

9.  Sample Preparation Requirements

|  | Samples are the single most critical gate before production. The drop test can only begin once all sample requirements below are fully met. Any shortcut in sample preparation invalidates the test result. |
| --- | --- |

| Rule | Requirement | Detail | Applies To |
| --- | --- | --- | --- |
| SR1 | Minimum sample count | 3 fully packed finished goods — 5 for high-risk products | All tests |
| SR2 | Final shipper carton | Must be included in every sample — production-grade carton only | All tests |
| SR3 | Inner box | Include if applicable to product configuration — confirm against BOM | All tests |
| SR4 | Cushioning material | EPE / Thermocol / Moulded pulp exactly as per approved spec — type and thickness must match | All tests |
| SR5 | Product inside | Actual fully assembled, functional product — no dummies or non-functional units | All tests |
| SR6 | Hand-cut samples | Permitted for initial validation only — GSM and paper spec must match frozen spec exactly | Initial only |
| SR7 | Production-line samples | Mandatory for final approval gate — hand-cut samples not accepted at this stage | Final approval |
| SR8 | Sample version tag | Each sample submission must carry a version number (e.g., V1, V2) and submission date | All tests |
| SR9 | Production-intent config | All packaging elements must be identical to the configuration that will be sent to production | All tests |

10.  Drop Test Procedure

The drop test is a mandatory pre-production certification exercise. No bulk order may be placed and no production may commence until a written PASS verdict has been issued and the specification has been frozen per Phase P8.

10.1 Test Conditions & Environment

| Parameter | Required Range | Unit | Pre-condition | Notes |
| --- | --- | --- | --- | --- |
| Drop Surface — Primary | Solid concrete floor | — | N/A | Flat, level, non-cushioned, free from debris |
| Drop Surface — Alt. | Steel impact plate | — | N/A | Flat, level, non-cushioned, free from debris |

10.2 Drop Sequence

As per Nuuk drop test standard(MII-PKG-SOP-001-NUUK-DT01)

10.3 Acceptance Criteria

All criteria below must be met. A single unmet criterion results in a FAIL verdict for that sample.

| # | Criterion | Verdict | Notes |
| --- | --- | --- | --- |
| C1 | No product damage of any kind (breakage, deformation) | PASS | Zero tolerance — any damage = FAIL |
| C2 | No functional failure on post-drop test | PASS | Full functional test mandatory — all product functions |
| C3 | No structural crack in product housing (including hairline cracks) | PASS | QM + Product team to inspect with torch |
| C4 | Carton remains fully closed — product not exposed | PASS | All closures, flaps, and tapes must hold |
| C5 | All seals intact (tape, glue, press-locks) | PASS | No partial seal failure acceptable |
| C6 | Internal components in correct position | PASS | No displacement that could cause damage in transit |
| C7 | Carton deformation — minor only | — | Acceptable only if no product protection impact |
| C8 | Cushion displacement — within acceptable limit | — | Measure and record (mm); document in checklist |
| C9 | Cosmetic carton denting — minor only | — | Acceptable — does not affect protective function |

10.4 Rejection Criteria

If any one of the following is observed, the sample is FAIL. Testing stops; NCR is raised immediately.

| # | Rejection Condition | Severity | Stop Test? | Immediate Action |
| --- | --- | --- | --- | --- |
| F1 | Product breakage or housing crack | CRITICAL | YES | Stop; raise NCR; notify QM + PM |
| F2 | Functional failure on post-drop test | CRITICAL | YES | Stop; raise NCR; notify Product team |
| F3 | Internal component displacement | CRITICAL | YES | Stop; raise NCR |
| F4 | Carton burst — product exposed | CRITICAL | YES | Stop; raise NCR |
| F5 | Total seal failure on all closures | HIGH | YES | Stop; raise NCR; notify QM |
| F6 | Severe crushing — transport / safety risk | HIGH | YES | Stop; document; raise NCR |

10.5 Post-Test Evaluation Checklist

Execute the following in sequence after all drops are complete. Do not skip steps.

| Step | Action | Method | Record Required | Owner |
| --- | --- | --- | --- | --- |
| E1 | Visual inspection of carton exterior | 360° physical examination — all 6 faces | Before & after photographs | QM Team |
| E2 | Open carton — inspect interior | Remove cushioning carefully; inspect all internal faces and insert panels | Photographic record | QM Team |
| E3 | Inspect cushioning alignment | Check position against approved spec drawing; measure any displacement | Displacement noted in mm on checklist | QM Team |
| E4 | Inspect product surface | Check for cracks, chips, scratches, deformation; use torch for hairline cracks | Photographic record of all findings | QM + Product Team |
| E5 | Perform full functional test | Power-on; operate all product functions per approved test plan | Pass / Fail noted for each function | Product Team |
| E6 | Record all observations | Complete signed test checklist; annotate photographs with finding labels | Signed and dated test checklist | QM Team |
| E7 | Issue formal PASS / FAIL verdict | Map all findings against Section 10.3–10.4; write verdict with justification | Written verdict report filed in QMS | QM + MII |

10.6 Iteration Process on Failure

|  | All iterations must be formally documented. No undocumented modifications are permitted at any stage. Each iteration loop (P5 → P6 → P7) must produce a new version-tagged sample and a new test report. The iteration log must record: failure mode, modification made, version number, re-test date, and verdict. |
| --- | --- |

| Step | Action | Owner | Output Required |
| --- | --- | --- | --- |
| I1 | Root Cause Analysis (RCA) — identify exact failure mode and component | QM + Project Lead | Written RCA report |
| I2 | Identify structural weakness — map failure location to specific packaging component | QM + Product Team | Failure mode documented in iteration log |
| I3 | Modify: Cushion thickness — increase EPE / foam thickness as per QM recommendation | Supplier + QM | Revised spec sheet issued |
| I4 | Modify: Flute type — upgrade 3-ply → 5-ply → 7-ply as required | Supplier + QM | Updated box spec with new ply count |
| I5 | Modify: GSM — increase liner paper GSM as required | Supplier + QM | Revised spec sheet |
| I6 | Modify: Fitment / corner protection — add corner inserts or internal partitions | Supplier + QM | Updated dieline / BOM |
| I7 | Build revised sample — assign new version number (e.g., V2, V3) | Supplier | Revised sample with version tag and date |
| I8 | Re-submit to QM — repeat Phases P5 → P6 → P7 | Project Lead + QM | New signed test report for each iteration |
| I9 | Repeat until all samples achieve PASS | QM Team | Final PASS report and iteration log filed |

10.7 Specification & Artwork Freeze

The following steps must be completed in sequence immediately after a PASS verdict is achieved. Bulk production cannot begin until all five steps are complete.

| Step | Action | Owner | Output |
| --- | --- | --- | --- |
| F1 | Specification Freeze — all box and cushioning parameters are locked; no further structural changes without full revalidation | QM + Product manager + MII project lead | Frozen spec sheet distributed to all stakeholders |
| F2 | Artwork Freeze — final artwork confirmed against approved dieline; version number assigned; file locked in QMS | BM + QM + PM | Locked artwork file with version reference |
| F3 | Issue Controlled Version Document — QM sign-off; version number; distributed to supplier, PM, QM | MII project lead + QM Manager | Signed controlled document on record |
| F4 | Transmit frozen spec and artwork to supplier in writing | MII project lead | Supplier written confirmation of receipt |
| F5 | Formal production release — PM and Project Lead confirm in writing; no verbal release accepted | PM + MII project lead | Written production release authorisation |

|  | REVALIDATION TRIGGERS — A full drop test revalidation cycle is mandatory if any of the following occur after freeze: • Supplier change (box or cushioning supplier) • Material change — GSM, PLY count, flute type, or cushion material • Design modification — any structural change to box, dieline, or cushioning layout • Complaint trend — recurring customer complaints indicating packaging damage in field |
| --- | --- |

11.  Approval Matrix (RACI)

|  | R = Responsible (executes) · A = Accountable (owns outcome) · C = Consulted · I = Informed |
| --- | --- |

| Activity | MII Project Lead | BM | QM | PM | Product Team | Supplier |
| --- | --- | --- | --- | --- | --- | --- |
| Requirement initiation | R/A | I | I | I | — | — |
| Artwork review & BM approval | R | A | C | A | — | — |
| Specification review & QM approval | R | C | A | A | — | C |
| Joint QM + BM approval (P4) | R | A | A | I | — | — |
| Sample preparation & submission | R | I | I | I | C | R |
| Testing | C | — | A | I | R | C |
| PASS / FAIL verdict | C | I | A | A | R | I |
| Iteration on failure | R | — | A | I | C | R |
| Specification freeze (F1–F5) | R | A | A | I | — | C |
| Supplier award & PO | R | I | I | C | — | C |
| Incoming material inspection | R | I | A | I | — | — |
| Tracker closure | R/A | I | I | I | — | — |

12.  Quality Checkpoints

12.1 Pre-Drop-Test Checklist (before Phase P6 begins)

|  | ✓ Artwork received and BM-approved (Phase P2 complete) ✓ Specification finalised and QM-approved (Phase P3 complete) ✓ Joint QM + BM approval confirmed; Column J = Approved (Phase P4 complete) ✓ Minimum 3 production-intent samples received with version tags (Phase P5 complete) ✓ Test environment conditioned for minimum 8 hours ✓ Drop surface confirmed: flat, level, non-cushioned, debris-free ✓ Video recording equipment ready; camera charged ✓ Signed test checklist form prepared |
| --- | --- |

12.2 Pre-Production Release Checklist (before Phase P9 begins)

|  | ✓ Written PASS verdict issued by QM (Phase P7 complete) ✓ Specification frozen and distributed (F1 complete) ✓ Artwork frozen with version number (F2 complete) ✓ Controlled version document issued and filed (F3 complete) ✓ Supplier has confirmed receipt of frozen spec and artwork in writing (F4 complete) ✓ Written production release authorised by PM and Project Lead (F5 complete) ✓ Tracker Column O (Final Approved Artwork) updated ✓ Tracker Remarks updated with spec freeze date |
| --- | --- |

12.3 Incoming Material Inspection Checklist (Phase P10)

|  | ✓ Quantity count matches purchase order exactly ✓ Box dimensions match frozen spec ±2mm tolerance ✓ Ply count and GSM verified by physical check or mill certificate ✓ Print quality: colour accuracy, bleed, and registration verified ✓ Sticker adhesion test performed on sample product surface ✓ No damage, moisture ingress, or deformation observed in transit ✓ Batch / lot number recorded for traceability ✓ Photographic record of sampled cartons retained |
| --- | --- |

13.  Timelines & Milestone Management

Packaging activities must be planned backwards from each product's launch window. The drop test adds 2–4 weeks to procurement lead time and must be factored into all project timelines.

13.1 2026 Product Launch Calendar

| Quarter | Month | Product / Category | Action Required |
| --- | --- | --- | --- |
| Q4 2025 / Q1 2026 | Jan – Mar | BRISK Lot 5, KRUSH v1 | Artwork + spec; drop test samples; PO placement |
| Q1 2026 | April | BFF v2, Kettle - widemouth | Finalise artwork and spec; QM & BM approval by March; sample submission |
| Q2 2026 | May – Jun | Halo v2 Connected, BRISK Mini, Krush v2, Krush Go, Ren Go, Ren Beam, Ren Pro | Samples submitted by April; drop test complete; PO released |
| Q2–Q3 2026 | Jul – Aug | Air Purifier, Kettle variants, Lit v3, Lit Pro, BFF | Upcoming list items actioned by Q1; samples ready by June |
| Q3 2026 | Sep – Oct | Heaters (Hotbox, Hot bar v2, Hot Dot, Hot Cube), Rover (2 SKUs), Styler | Artwork + spec initiated in Q2; drop test by August |

13.2 Standard Lead Times (including drop test)

| Activity | Duration | Buffer | Notes |
| --- | --- | --- | --- |
| Artwork revision cycle | 2 working days per round | Allow 2 rounds | Start early — do not wait until spec is ready |
| QM + BM approval cycle | 2 working days | Concurrent review | Submit artwork and spec simultaneously |
| Sample preparation by supplier | 7 working days | 2 days | Confirm with supplier at PO stage |
| Drop test conditioning + execution | 2 working days | 1 day | Per sample set; excludes iteration time |
| Iteration cycle (if FAIL) | 7 working days per loop | Allow 2 loops | Factor into timeline from project start |
| Corrugated outer box (standard) | 15–20 working days | 5 days | Post spec freeze and PO |
| Stickers / labels (standard) | 7–10 working days | 3 days | Post spec freeze and PO |

14.  Tracker Maintenance & Data Integrity

|  | The Packaging Tracker is the operational record for this SOP. All rules below are mandatory. R1 Update within 24 hours of any status change or material event. R2 Status = 'Close' only after physical receipt is confirmed and QM inspection has passed. R3 Available Qty reflects actual on-hand stock only — never projected, in-transit, or ordered quantities. R4 All remarks must include a date stamp — e.g., 'Sample PASS drop test 14-Mar-2026'. R5 Never delete rows — cancelled items: add remark 'CANCELLED — [reason]' and set Status = Close. R6 Column J (QM & BM Approval) must only be set to Approved after both QM and BM have independently confirmed. R7 Column O (Final Approved Artwork) must only be confirmed after spec freeze is complete. R8 Weekly review of all Open items every Monday by the Project Lead. R9 Once actioned, Upcoming Project List items must be migrated to the main tracker. R10 Drop test outcome must be logged in Remarks: 'Drop test PASS V[n] — [date]' or 'Iteration [n] FAIL — [root cause]'. |
| --- | --- |

15.  Escalation Matrix

| Situation | First Escalation | Second Escalation | SLA |
| --- | --- | --- | --- |
| Artwork not received — blocking approval | BM(Packaging lead) | BM(Manager) | After 3 working days |
| QM or BM approval pending | QM / BM (Lead) | QM / BM (Manager) | After 5 working days |
| Samples not submitted by supplier | MII project lead | MII project lead | After 5 working days from agreed date |
| Drop test FAIL — no root cause identified | QM(lead) + MII project lead | QM + PM + MII manager(operations) | Within 1 working day of verdict |
| Iteration loop exceeds 3 rounds | PM(Manager) + QM(Manager) | MII head / Senior Leadership | Immediately after 3rd FAIL |
| Spec freeze delayed post-PASS | QM lead | QM manager | Within 2 working days of PASS |
| Supplier unable to meet production lead time | MII project lead | MII operations manager | Immediately |
| Incoming bulk material rejected (NCR raised) | MII project lead | MII operations manager | Same day as rejection |

16.  Key Suppliers & Document Control

16.1 Key Suppliers & Material Mapping

| Supplier | Material Type | Products Served |
| --- | --- | --- |
| Dhawan Packaging Cliffton Packaging | Corrugated outer boxes (brown, white, black) | All MII products + Spare boxes + Accessory boxes |
| C.V Print Dhawan Packaging Saint Krafts | Stickers, labels, printed materials | All MII products + Spare boxes + Accessory sticker |
| E-Pack, Westan, Elin, Zetwerk, Anand shakti | Receiving, warehousing, IQC and inspection | All products |

16.2 Mandatory Documentation — Complete Record Set

| # | Document | Format | Filed Where | Retention |
| --- | --- | --- | --- | --- |
| D1 | Drop test video recording (all sessions) | MP4 / MOV | Project folder / QMS | Permanent |
| D2 | Before & after photographs | JPEG / PNG | Project folder / QMS | Permanent |
| D3 | Signed test checklist (all iterations) | Excel / PDF | Project folder / QMS | Permanent |
| D4 | Deviation notes (if any) | PDF / Word | QMS | Permanent |
| D5 | Final PASS verdict report | PDF (signed) | QMS | Permanent |
| D6 | Iteration log (if applicable) | Excel | Project folder | Permanent |
| D7 | Frozen spec sheet | PDF | QMS + Supplier | Permanent |
| D8 | Locked artwork file | AI / PDF | QMS + Supplier | Permanent |
| D9 | Controlled version document (signed) | PDF | QMS | Permanent |
| D10 | Written production release authorisation | Email / PDF | Project folder | Permanent |

16.3 Revision History

| Version | Date | Author | Reviewed By | Description |
| --- | --- | --- | --- | --- |
| 1.0 | March 2026 | MII team + QM team | QM + PM + BM | MII Packaging SOP |

This document is subject to review every 6 months or upon any material change in the packaging approval process, supplier base, product portfolio, or applicable ISTA standards. All amendments require QM and BM sign-off. Contact the Process Owner for any queries.