STANDARD OPERATING PROCEDURE Packaging & Transit Evaluation Drop Test • Stair Roll Test • Vibration Test • Stack Test Home Appliance Manufacturing — Dispatch Quality Assurance

| Document No. | Revision | Effective Date | Review Date |
| --- | --- | --- | --- |
|  | Rev 00 |  |  |
| Department | Prepared By | Reviewed By | Approved By |
| Quality Assurance | Shivesh Thakral |  | Kartik Gupta |

NOTE: This SOP  governs the Packaging & Transit Evaluation process conducted by the Nuuk QA team to verify that home appliance products and their packaging are capable of withstanding real-world transit and handling conditions before dispatch to customers. All tests are based on ISTA-1A and ISTA-6A standards with Nuuk-specific drop heights defined for Indian dispatch conditions.

# 1. Purpose

This SOP defines the systematic process for conducting Packaging and Transit Evaluation tests on finished home appliance units prior to dispatch. The objective is to verify that the product packaging is robust enough to protect the unit against physical shocks, drops, stair-rolling, vibration, and stacking loads that are typically encountered during storage, transport, and last-mile delivery. The Nuuk QA team performs these tests as part of the dispatch quality gate to ensure products reach customers in full working condition without cosmetic or functional damage.

# 2. Scope

This SOP applies to:

- All home appliance models dispatched from the Indian partner’s facility to customers or distribution centres

- New packaging designs prior to approval for use in production

- Revised packaging iterations (Iteration 1, Iteration 2, etc.) where any packaging parameter has changed

- The Nuuk QA team responsible for conducting, recording, and approving transit evaluation tests

- The Brand Representative team responsible for reviewing and approving final test results and packaging specifications

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Nuuk QA Member(s) | Conduct all transit evaluation tests (Drop, Stair Roll, Vibration, Stack) as per this SOP. Record all observations and images. Fill the Transit Evaluation Report. Raise NCR if any test fails. |
| Nuuk QA Lead | Review test results and Transit Evaluation Report. Provide pass/fail verdict on packaging design. Escalate packaging failures to the partner. Approve repeat test after packaging rework. |
| Partner’s QC Team | Provide packed product samples for testing. Ensure packaging specifications (plies, GSM, flute type, bursting strength) are documented per the Packaging Specs sheet. Witness testing when required. |
| Packaging Partner / China Partner | Submit CAPA for any packaging failure within defined timelines. Provide revised packaging for re-evaluation. |

# 4. Abbreviations

| Term | Full Form |
| --- | --- |
| ISTA | International Safe Transit Association |
| ISTA-1A | ISTA Test Procedure 1A — Packaged-Product for Parcel Delivery |
| ISTA-6A | ISTA Test Procedure 6A — Amazon.com Packaged-Products for Shipment |
| OQC | Outgoing Quality Check |
| PDI | Pre-Delivery Inspection |
| NCR | Non-Conformance Report |
| CAPA | Corrective and Preventive Action |
| GSM | Grams per Square Metre (paper weight) |
| L*W*H | Length × Width × Height (box dimensions) |
| WI | Work Instruction |

# 5. Pre-Test Requirements

## 5.1  Packaging Specification Documentation

Before any transit evaluation test is conducted, the following packaging details must be documented in the Packaging Specifications record (per the Packaging Specs sheet):

| Parameter | Unit Box | Outer Box |
| --- | --- | --- |
| No. of Plies | Record for each iteration | Record for each iteration |
| GSM | Record for each iteration | Record for each iteration |
| Flute Type | Record for each iteration | Record for each iteration |
| Bursting Strength (kg/cm²) | Record for each iteration | Record for each iteration |
| Bursting Factor | Record for each iteration | Record for each iteration |
| Box Dimensions (L×W×H) | Record for each iteration | Record for each iteration |

This record must be completed for each packaging iteration (Iteration 1, Iteration 2, etc.) being evaluated. No test shall commence without the packaging specification on record.

## 5.2  Sample Selection

- Minimum 1 packed unit per packaging iteration to be used for each test type.

- The sample must be a production-representative packed unit — assembled, packed, and sealed exactly as it would be dispatched.

- Record: Product name, model name, model code, team involved, test date, box dimensions, box weight, drop height (as applicable), and packaging iteration.

- Photograph the packaging and product before the test (pre-test images for both packaging and product).

## 5.3  Test Environment & Equipment

- Tests to be conducted at ambient room temperature unless otherwise specified.

- Drop tests: Use a calibrated drop test platform or a measured drop height marked on a rigid wall surface. Ensure free-fall with no lateral obstruction.

- Stair roll tests: Use a standard staircase with stair height as specified in Section 6.3.

- Stack tests: Use calibrated weights or evenly distributed load applied on top of the packed sample.

- Vibration tests: Use a calibrated vibration table (random or sinusoidal as applicable).

# 6. Test Procedures

## 6.1  Drop Test (ISTA-1A & ISTA-6A Combined)

The drop test simulates the shocks a packaged product experiences during transit, loading, and unloading. Two drop sequences are applied — ISTA-1A (10 drops) and ISTA-6A (17 drops) — on the same sample unit. Nuuk has defined higher drop heights than the ISTA standards to better reflect Indian last-mile delivery conditions.

Drop Height Reference (Based on Product Weight)

| Product Weight | Nuuk Drop Height (Use This) | ISTA-1A Height | ISTA-6A Height |
| --- | --- | --- | --- |
| 1 – 9 kg | 100 cm | 76 cm | 91 cm |
| 10 – 18 kg | 80 cm | 61 cm | 76 cm |
| 19 – 27 kg | 60 cm | 46 cm | 61 cm |
| 28 – 45 kg | 50 cm | 30 cm | 46 cm |
| 46 – 68 kg | 30 cm | 20 cm | 30 cm |

Important: Nuuk QA shall always use the “Nuuk Drop Height” column for all drop tests. The ISTA heights are provided for reference only.

ISTA-1A Drop Sequence (10 Drops)

| Seq. | Orientation | Specific Face / Edge / Corner |
| --- | --- | --- |
| 1 | Corner | Most fragile face — 3 corners. If not known, test corners 2-3-5 |
| 2 | Edge | Shortest edge radiating from the corner tested |
| 3 | Edge | Next longest edge radiating from the corner tested |
| 4 | Edge | Longest edge radiating from the corner tested |
| 5 | Face | One of the smallest faces |
| 6 | Face | Opposite small face |
| 7 | Face | One of the medium faces |
| 8 | Face | Opposite medium face |
| 9 | Face | One of the largest faces |
| 10 | Face | Opposite large face |

ISTA-6A Drop Sequence (17 Drops)

| Seq. | Orientation | Specific Face / Edge / Corner |
| --- | --- | --- |
| 1 | Face | Edge 3-4 |
| 2 | Face | Edge 3-6 |
| 3 | Face | Edge 4-6 |
| 4 | Face | Corner 3-4-6 |
| 5 | Face | Corner 2-3-5 |
| 6 | Face | Edge 2-3 |
| 7 | Edge | Edge 1-2 |
| 8 | Edge | Face 3 |
| 9 | Corner | Face 3 |
| 10 | Corner | Edge 3-4 |
| 11 | Face | Edge 3-6 |
| 12 | Face | Edge 1-5 |
| 13 | Face | Corner 3-4-6 |
| 14 | Face | Corner 1-2-6 |
| 15 | Face | Corner 1-4-5 |
| 16 | Face | Face 3 |
| 17 | Face | Face 3 on Hazard |

Drop Test Procedure Steps

- Record product weight. Refer to the Nuuk Drop Height table above to determine the correct drop height.

- Record box dimensions (L×W×H), box weight, packaging iteration, and drop height on the test record.

- Photograph the packaging and product before the test (pre-drop images — all four sides of packaging and product exterior).

- Position the sample at the correct drop height. Ensure free-fall with no lateral push or support.

- Execute all 10 drops in the ISTA-1A sequence in order, followed by all 17 drops in the ISTA-6A sequence.

- After all drops are complete, photograph the packaging and product (post-drop images — packaging exterior, packaging interior/cushioning, product exterior).

- Perform a functional check on the product immediately after the drop sequence.

- Record results against each acceptance criterion.

- Record remarks and declare the final verdict (Satisfactory / Not Satisfactory).

Acceptance Criteria (applicable to all tests):

| Parameter | Requirement | Result Field |
| --- | --- | --- |
| Packaging | Packaging shall not be damaged / burst / deformed beyond acceptable limits. | Pass / Fail |
| Product Damage | Products should not have any external or internal damage. | Pass / Fail |
| Functional Check | The product should work normally under standard use conditions after the test. | Pass / Fail |
| Aesthetics | No gaps on mating parts; no scratches on body beyond acceptable cosmetic limits. | Pass / Fail |

Verdict: Satisfactory — if all 4 parameters pass. Not Satisfactory — if any parameter fails; raise NCR immediately.

## 6.2  Vibration Test

The vibration test simulates the sustained vibrations a packaged product is exposed to during road or rail transport. The test is conducted on a vibration table using ISTA-recommended vibration profiles.

Vibration Test Procedure Steps

- Place the packed product sample on the vibration table in its normal dispatch orientation (top face up).

- Apply the vibration profile as per ISTA-1A or ISTA-6A random vibration specification for the applicable product weight range.

- Conduct the test for the full duration specified in the vibration profile.

- After the test, inspect the packaging and product. Photograph post-vibration state.

- Perform a functional check on the product.

- Record results and verdict.

Note: If a calibrated vibration table is not available at the test facility, the Nuuk QA Lead must note this and schedule the vibration test at an approved external testing laboratory. The vibration test shall not be skipped.

Acceptance Criteria (applicable to all tests):

| Parameter | Requirement | Result Field |
| --- | --- | --- |
| Packaging | Packaging shall not be damaged / burst / deformed beyond acceptable limits. | Pass / Fail |
| Product Damage | Products should not have any external or internal damage. | Pass / Fail |
| Functional Check | The product should work normally under standard use conditions after the test. | Pass / Fail |
| Aesthetics | No gaps on mating parts; no scratches on the body beyond acceptable cosmetic limits. | Pass / Fail |

Verdict: Satisfactory — if all 4 parameters pass. Not Satisfactory — if any parameter fails; raise NCR immediately.

## 6.3  Stair Roll Test

The stair roll test simulates the product being accidentally rolled, dropped, or slid down a staircase during last-mile delivery — a common occurrence in Indian residential delivery scenarios. This test is applicable only for products in the 1–9 kg weight range.

Stair Roll Drop Sequence (6 Steps)

| Seq. | Orientation | Specific Face / Edge / Corner Description |
| --- | --- | --- |
| 1 | Face | Bottom face leading down stairs (face drop on first stair) |
| 2 | Edge | Bottom leading edge contacts first stair |
| 3 | Corner | Bottom leading corner contacts first stair |
| 4 | Face | Rotate 90° — side face leading down stairs |
| 5 | Edge | Side leading edge contacts first stair |
| 6 | Corner | Side leading corner contacts first stair |

Stair Parameters (by Product Weight)

| Product Weight | Stair Height | Number of Stairs |
| --- | --- | --- |
| 1 – 9 kg | 20 cm | 10 stairs |
| 10 – 18 kg | NA | NA |
| 19 – 27 kg | NA | NA |
| 28 – 45 kg | NA | NA |
| 46 – 68 kg | NA | NA |

Note: Stair roll test is only applicable for products weighing 1–9 kg. For heavier products, this test is marked NA and is not required.

Stair Roll Test Procedure Steps

- Confirm product weight. If 10 kg or above, mark the Stair Roll Test as NA and proceed to the next test.

- Record stair height (20 cm per stair), number of stairs (10), box dimensions, box weight, and packaging iteration.

- Photograph packaging and product before the test (pre-stair roll images).

- Execute all 6 orientations in sequence. For each orientation, allow the product to roll down the full 10 stairs under its own momentum — no external force to be applied after the initial placement.

- After all 6 orientations, photograph packaging and product (post-stair roll images).

- Perform a functional check on the product.

- Record results and verdict.

Acceptance Criteria (applicable to all tests):

| Parameter | Requirement | Result Field |
| --- | --- | --- |
| Packaging | Packaging shall not be damaged / burst / deformed beyond acceptable limits. | Pass / Fail |
| Product Damage | Products should not have any external or internal damage. | Pass / Fail |
| Functional Check | The product should work normally under standard use conditions after the test. | Pass / Fail |
| Aesthetics | No gaps on mating parts; no scratches on the body beyond acceptable cosmetic limits. | Pass / Fail |

Verdict: Satisfactory — if all 4 parameters pass. Not Satisfactory — if any parameter fails; raise NCR immediately.

## 6.4  Stack Test

The stack test simulates the compressive load a packaged product experiences when other packages are stacked on top of it during warehousing or transport. The test evaluates both the structural integrity of the packaging and any potential damage to the product inside.

Stack Test Sequence (4 Orientations)

| Seq. | Orientation | Description |
| --- | --- | --- |
| 1 | Top Face | Normal stacking orientation — maximum load applied |
| 2 | Top Face | Normal stacking orientation — 50% load applied |
| 3 | Bottom Face | Inverted stacking — maximum load applied |
| 4 | Side Face | Side stacking orientation — maximum load applied |

Stack Load Parameters (by Product Weight)

| Product Weight | Applied Load | Duration |
| --- | --- | --- |
| 1 – 9 kg | 2× product weight | 24 hours |
| 10 – 18 kg | 2× product weight | 24 hours |
| 19 – 27 kg | 2× product weight | 24 hours |
| 28 – 45 kg | 1.5× product weight | 24 hours |
| 46 – 68 kg | 1.5× product weight | 24 hours |

Stack Test Procedure Steps

- Determine product weight. Calculate the required applied load (2× or 1.5× product weight as per the table above).

- Record applied load, box dimensions, box weight, and packaging iteration.

- Photograph packaging and product before the test (pre-stack images).

- Place the sample in Orientation 1 (top face, normal stacking). Apply the maximum load uniformly across the top surface. Maintain for 24 hours.

- After 24 hours, inspect and photograph. Record observations immediately.

- Repeat for Orientations 2, 3, and 4 using a fresh sample if needed (or the same sample if structural integrity allows).

- Perform a functional check on the product after all orientations are complete.

- Record results and verdict.

Acceptance Criteria (applicable to all tests):

| Parameter | Requirement | Result Field |
| --- | --- | --- |
| Packaging | Packaging shall not be damaged / burst / deformed beyond acceptable limits. | Pass / Fail |
| Product Damage | Products should not have any external or internal damage. | Pass / Fail |
| Functional Check | The product should work normally under standard use conditions after the test. | Pass / Fail |
| Aesthetics | No gaps on mating parts; no scratches on the body beyond acceptable cosmetic limits. | Pass / Fail |

Verdict: Satisfactory — if all 4 parameters pass. Not Satisfactory — if any parameter fails; raise NCR immediately.

# 7. Recording, Photography & Reporting

## 7.1  Photography Requirements

Photographs are mandatory at the following stages for every test:

| Test Stage | Pre-Test Photos Required | Post-Test Photos Required |
| --- | --- | --- |
| Drop Test | All 4 sides of packaging + product exterior (all angles) | Packaging exterior, interior cushioning, product exterior, any damage close-ups |
| Vibration Test | Packaging + product exterior | Packaging + product exterior + any loosened internal parts |
| Stair Roll Test | All 4 sides of packaging + product exterior | Packaging exterior, interior, product exterior, any damage close-ups |
| Stack Test | Packaging + product before load is applied | Packaging post-load (immediately after removal), product exterior |

## 7.2  Transit Evaluation Report

The Nuuk QA member shall complete one Transit Evaluation Report per packaging iteration tested. The report must include:

- Product details: Product name, Model name, Model code, Team involved, Test date

- Packaging specifications (from Packaging Specs sheet): No. of plies, GSM, flute type, bursting strength, bursting factor, box dimensions — for both unit box and outer box

- Test-specific parameters: Box dimensions, box weight, drop height / stair height / applied load / vibration profile (as applicable)

- Pre-test and post-test photographs (labelled clearly)

- Results table filled for each test: Packaging, Damages, Function, Aesthetics — Pass or Fail

- Remarks for any observation, even if result is Pass

- Final verdict per test: Satisfactory or Not Satisfactory

- Prepared By and Approved By fields, both signed

## 7.3  NCR Raising on Test Failure

If any test returns a “Not Satisfactory” verdict, the Nuuk QA member shall:

- Raise an NCR immediately, referencing the test type, product model, packaging iteration, and the specific parameter that failed.

- Inform the Nuuk QA Lead.

- Quarantine the failed packaging iteration — no units packed in this packaging shall be dispatched.

- Request CAPA from the packaging supplier / China partner as per SOP-DA-003.

- After CAPA and packaging revision, conduct a full repeat of the failed test on the revised packaging (new iteration). Partial re-test is not acceptable.

# 8. Test Summary — Quick Reference

| Test | Standard | No. of Drops/Steps | Key Parameter | Applicable Weight |
| --- | --- | --- | --- | --- |
| Drop Test | ISTA-1A + ISTA-6A (Combined) | 10 (ISTA-1A) + 17 (ISTA-6A) = 27 total drops | Nuuk drop height (100–30 cm by weight) | 1–68 kg |
| Vibration Test | ISTA-1A / ISTA-6A | Continuous per vibration profile | Random vibration per ISTA profile | 1–68 kg |
| Stair Roll Test | ISTA / Nuuk Custom | 6 orientations × 10 stairs each | Stair height: 20 cm; 10 stairs | 1–9 kg only |
| Stack Test | ISTA-1A / ISTA-6A | 4 orientations | Load: 1.5–2× product weight for 24 hrs | 1–68 kg |

# 9. Records & Documentation

- Packaging Specifications Sheet — one per packaging iteration, retained per product model

- Transit Evaluation Report (filled Excel / printed format) — retained per test event

- Pre-test and post-test photographs — archived per test event

- NCR Log — for any test failure, retained for minimum 3 years

- CAPA records (per SOP-DA-003) — retained per NCR

- Brand Representative Approval Email — archived per packaging iteration approved

# 10. SOP Review & Version Control

This SOP shall be reviewed annually or on any of the following triggers:

- Change in product model weight, dimensions, or packaging design

- Introduction of new dispatch route or logistics partner

- Recurring packaging failure during dispatch

- Update to ISTA test standards

- Request from Brand Representative