STANDARD OPERATING PROCEDURE Work Instructions & Operator Training Assembly, Packaging & Quality Inspection — Home Appliance Manufacturing

| Document No. | Revision | Effective Date | Review Date |
| --- | --- | --- | --- |
|  | Rev 00 |  |  |
| Department | Prepared By | Reviewed By | Approved By |
| Quality Assurance | Shivesh Thakral |  | Kartik Gupta |

# 1. Purpose

This Standard Operating Procedure (SOP) defines the requirements for developing, approving, and maintaining Work Instructions (WI) for all assembly, packaging, and quality inspection activities carried out at the partner's facility, and the standards for training operators on those Work Instructions. It ensures that every operator performing a critical or standard assembly task does so consistently, correctly, and with full awareness of quality expectations — including past defect history — before they are deployed on the production line.

# 2. Scope

This SOP applies to:

- All assembly, packaging, in-process quality inspection, and electrical safety testing operations for home appliance products carried out at the partner's facility.

- The partner's production and QC team responsible for creating, maintaining, and training to Work Instructions.

- The Nuuk QA team responsible for reviewing, approving WI, and verifying training compliance during process audits. Nuuk QA team can also visit the partner and provide trainings for Customer specific requirement if needed.

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Partner | Develop and maintain Work Instructions for all assembly, packaging, OQC, and critical quality inspection stations. Conduct training for all operators before deployment on the line. Maintain signed training records. Update WI when process changes occur and submit revised WI to Nuuk for re-approval. |
| Nuuk QA Team | Review and approve all WI submitted by the partner before production begins. Verify during process audits that operators are working as per approved WI and Control Plan. Provide defect history to the partner to be included in operator training. Visit the partner's facility when required to conduct CTQ assembly training directly. |
| Nuuk QA Lead | Final approval authority for all WI. Conduct or supervise process audits to verify WI compliance and training adequacy. Decide on production go / no-go if critical WI are missing or operators at CTQ stations are untrained. Share defect history and known quality issues with the partner team for inclusion in operator training material. Review process audit findings related to WI compliance and escalate non-compliance to the partner. |

# 4. Reference Documents

- SOP — Incoming Quality Control (SKD Kits)

- SOP — In-Process Quality Control (Assembly)

- Approved Control Plan — Assembly Operations (CTQ stations)

- Process Audit / Line Patrol Inspection Report

- Production WI format

- Operator Training Record Form

- CTQ Operator Competency List

- Defect History Report (issued by Brand / Nuuk team)

# 5. Abbreviations

| Term | Full Form |
| --- | --- |
| WI | Work Instruction |
| CTQ | Critical to Quality — operations directly impacting product safety, performance, or compliance |
| CTP | Critical to Process — process parameters that must be controlled to ensure CTQ outputs |
| OQC | Operational Quality Check |
| ESD | Electrostatic Discharge |
| BOM | Bill of Materials |
| CAPA | Corrective and Preventive Action |
| NCR | Non-Conformance Report |

# 6. Detailed Procedure

## 6.1 Work Instruction — Coverage Requirements

The partner shall develop and maintain a Work Instruction (WI) for every activity listed below. No operator shall perform any of these activities without a Nuuk-approved WI being available and displayed at the station.

| S.No. | Activity Type | Examples | WI Required? |
| --- | --- | --- | --- |
| 1 | Assembly Operations — General | Body screw fitment, bucket assembly, top cover fixing | Yes — Mandatory |
| 2 | CTQ / Critical Assembly Operations | Power cord crimping & connection, PCB fitment, PCB/JST connector gluing, wire routing, torque-critical fastening | Yes — Mandatory (CTQ flag on WI) |
| 3 | Electrical Safety Testing | ECR test, HV test, IR test, leakage current test, hi-pot, ageing / soaking test | Yes — Mandatory (CTQ flag) |
| 4 | Functional Testing & OQC | Power-on test, mode/function check, VAW meter test, cooling stage check | Yes — Mandatory |
| 5 | Sticker / Label Application | Rating label, ceramic tech sticker, serial number sticker, QR / barcode sticker | Yes — Mandatory |
| 6 | Packaging Operations | Inner packing (EPS, polybag), unit carton packing, box printing (month/year), master box packing, BOPP tape sealing | Yes — Mandatory |
| 7 | In-Process Quality Inspection | Patrol inspection checks, OQC steps, first-off inspection, repair / rework handling | Yes — Mandatory |
| 8 | ESD Handling | PCB handling, wristband use, ESD mat grounding checks | Yes — Mandatory |

Note: WI for CTQ operations must be clearly marked with a 'CTQ' flag on the header. These receive priority review and any change to a CTQ WI requires re-approval by the Nuuk QA Lead before the revised WI is deployed on the line.

## 6.2  Work Instruction — Format & Content Requirements

All WI submitted by the partner to Nuuk for approval shall include the following sections as a minimum:

| S.No. | WI Section | Required Content |
| --- | --- | --- |
| 1 | Header Block | Document number, revision number, effective date, product name, model number, department, assembly line name, station / stage name |
| 2 | CTQ / CTP Flag | Clearly identify whether the station is CTQ (Critical to Quality) or CTP (Critical to Process). CTQ stations must be highlighted prominently on the WI header. |
| 3 | Process Flow / Operation Steps | Step-by-step assembly or inspection instructions in both English and Hindi (bilingual). Each step numbered. Use simple language understandable by shop-floor operators. |
| 4 | Process Operation Diagram / Image | Visual photograph or diagram showing the correct assembly position, component location, wire routing, or result. Reference images for OK and NG conditions where applicable. |
| 5 | Inspection Items & Specs | Inspection item name, specification / acceptance criteria, inspection method (Visual / Torque meter / Instrument), and defect category (Critical / Major / Minor) |
| 6 | Machine / Tools / Parameters | Tool name, specification (e.g. torque range: 3–6 kgf/cm²), and quantity required at the station |
| 7 | List of Parts | Part name and quantity consumed at this station, as per BOM |
| 8 | Defect / Issue History | Known issues or defects previously encountered at this station (provided by Brand / Nuuk). Operators must be made aware of these during training. |
| 9 | Revision History | Revision number, date, change description, reason for change (e.g. 'Issue found in PDI inspection') |

Note: The WI shall be bilingual (English and Hindi) so that all operators — regardless of language preference — can follow the instructions without ambiguity. Visual diagrams and photographs are mandatory for all assembly and CTQ stations.

## 6.3  Work Instruction — Submission & Approval Process

- The partner prepares the WI for each station and submits the full set to the Nuuk QA team via email before the first production run of a new model, or before any revised WI is put into use on the line.

- The Nuuk QA team reviews each WI against the following criteria:

- Correctness of assembly / inspection steps vs. approved engineering drawings and Control Plan

- Presence of CTQ flag on relevant stations

- Completeness of tool specifications, torque values, and part list

- Adequate visual references (OK / NG photos or diagrams)

- Bilingual content (English + Hindi)

- Defect / issue history included where applicable

- The Nuuk QA Lead approves or returns the WI with review comments within 5 working days of receipt.

- If comments are raised, the partner incorporates them and resubmits. No production shall begin until all WI for the model are approved.

- Approved WI are assigned a document number and revision number. The partner prints the approved WI and displays them at the respective stations before the line setup.

- Any change to an approved WI (process change, tool change, component revision, or CAPA-driven update) requires re-submission to Nuuk for approval before the updated WI is deployed.

## 6.4  Work Instruction — Display at Station

- The approved WI (latest revision) shall be printed and laminated (or placed in a protective sleeve) and displayed at the assembly or inspection station it covers.

- WI must be visible to the operator while they are performing the operation — not folded away in a drawer.

- If a station covers multiple operations (as in some multi-step WI), all relevant pages must be displayed.

- Outdated WI revisions must be removed from the station and archived. Only the current approved revision shall be present at the station.

## 6.5  Operator Training Requirements

The partner shall ensure that every operator is formally trained on the WI for each station they will work on before they are deployed. Training is not optional and must be completed and documented for every operator, including new joiners, operators who change stations, and operators returning after an extended absence (> 30 days).

6.5.1  Training Coverage

Every training programme for assembly operators shall cover the following topics at minimum:

| S.No. | Training Topic | Content | Who Delivers |
| --- | --- | --- | --- |
| 1 | Product & Part Knowledge | Overview of the home appliance being assembled: product name, model, main components, their function, and correct part identification. Operators must be able to distinguish components by name. | Partner QA / Production Supervisor |
| 2 | Work Instruction Walkthrough | Step-by-step walkthrough of WI for each station the operator will work on. Emphasis on correct sequence, tool usage, torque values, and inspection criteria. | Partner QA / Production Supervisor |
| 3 | CTQ Assembly Operations | Detailed hands-on training on all CTQ stations: power cord crimping, PCB fitment, wire connection, torque-critical screwing, JST glue application, wiring routing. Operators must demonstrate correct execution before being cleared for CTQ work. | Partner QA Lead + Nuuk QA (for critical models) |
| 4 | Defect & Issue History Awareness | Training on previously identified defects at each station: what the defect looks like, why it occurs, and how to prevent it. Defect history shared by Brand / Nuuk team is included in this module. | Partner QA + Nuuk QA (provides defect history) |
| 5 | Packaging Training | Correct inner packing (EPS orientation, polybag placement, accessories inclusion), unit carton packing, BOPP tape application, master box stickering, and carton marking / printing. | Partner QA / Production Supervisor |
| 6 | Quality Inspection Basics | How to identify Major, Minor, and Critical defects. Visual standards. Use of inspection instruments (torque meter, ECR meter, HV tester, etc.). Segregation and tagging of non-conforming units. | Partner QA + Nuuk QA |
| 7 | ESD & Safety Awareness | Importance of ESD protection when handling PCBs and electrical components. Correct wristband usage, ESD mat grounding checks, and electrical safety awareness on the line. | Partner QA / Production Supervisor |

6.5.2  Defect History in Training

The Nuuk QA team shall share the relevant defect history — including photos where available — with the partner for inclusion in operator training material. This includes:

- Defects found during IQC, IPQC patrol, OQC, or final inspection for that product model

- Defects reported from PDI (Pre-Delivery Inspection) or customer returns

- Issues that triggered CAPA from previous production lots

Operators must be shown what these defects look like and specifically told what actions at their station can cause or prevent each defect. This awareness directly reduces recurrence of known issues.

6.5.3 Nuuk QA On-Site Training (When required)

In cases where a new model is introduced, a CTQ process is particularly complex, or repeated issues have been found at a specific station, the Nuuk QA Lead (or a designated QA member) shall visit the partner's facility and conduct hands-on training for the operators at the relevant stations. This visit-based training shall:

- Be planned in advance and communicated to the partner with a training agenda

- Focus specifically on CTQ operations — correct assembly method, common mistakes, quality checkpoints

- Include live demonstration followed by supervised practice by the operators

- Be documented with an attendance sheet signed by each operator trained

- Result in updated training records that are filed by the partner and available for audit

## 6.6 Training Records & Competency Documentation

The partner shall maintain a formal training record for every operator. The training record must include:

- Operator name, employee ID, and designation

- List of stations / operations the operator is trained for

- Training date and trainer name (partner trainer, or Nuuk QA if on-site training was provided)

- Operator's sign-off confirming they have been trained and understood the WI and quality requirements

- Trainer's sign-off confirming training completion

- Assessment result — where applicable (especially for CTQ stations), a brief practical assessment is conducted before an operator is cleared to work independently

Training records shall be maintained in a Training Register at the partner's facility and made available immediately when requested during a process audit. For CTQ operations, the operator's name must also appear on the CTQ Operator Competency List which is shared with and acknowledged by the Nuuk QA team.

## 6.7 Process Audit — WI & Training Verification

During process audits conducted by the Nuuk QA Lead (Section 7.5 and the Process Audit / Line Patrol Inspection Report), the following WI and training compliance checks are carried out at the partner's assembly facility:

| S.No. | Audit Check Point | Acceptable Condition | Finding Category |
| --- | --- | --- | --- |
| 1 | Approved WI displayed at every assembly and inspection station | Latest approved revision visible and legible at the station | Major |
| 2 | Operator working as per WI sequence and method | No deviation from WI steps; correct tool and technique used | Critical (CTQ station) / Major (other) |
| 3 | CTQ station manned by trained and qualified operator | Operator name present on CTQ Competency List; training record available | Critical |
| 4 | Training records available for all operators on the line | Signed training record for each operator covering all stations they operate | Major |
| 5 | Torque tools set to correct specified values | Tool setting matches WI specification; calibration tag valid | Critical / Major |
| 6 | Control Plan being followed at inspection points | Sample size, frequency, and inspection method per Control Plan | Major |
| 7 | WI revision is current — no outdated WI in use | WI at station matches latest approved revision in Nuuk system | Major |
| 8 | Defect history / known issues communicated to operators | Operators can recall key defects for their station when asked | Minor |
| 9 | ESD protection in use at electrical/PCB stations | Wristbands worn, mats grounded; resistance within spec | Critical |
| 10 | Non-conforming units tagged and segregated correctly | NG units in Hold bin with red tag; not mixed with OK output | Major |

Audit findings are categorised as Critical, Major, or Minor and recorded in the Process Audit Report. Dispositions:

- Critical finding: Production at the affected station is stopped immediately. The issue must be resolved before work resumes.

- Major finding: Corrective action must be implemented within 24 hours. Nuuk QA follows up before the next production shift.

- Minor finding: Partner acknowledges and commits to corrective action within 3 working days.

All audit findings, corrective actions, and their closure status are documented in the Process Audit Report and filed by the Nuuk QA team.

## 6.8  WI Update Triggers & Revision Management

The partner must initiate a WI revision and submit to Nuuk for re-approval under any of the following conditions:

- A CAPA raises a need to change the assembly method, tool, or inspection criterion at a station

- A new defect is identified that requires an additional inspection step or process control

- A component change from the Partner alters the assembly method at a station

- Regulatory or certification requirements change (e.g., BIS, CE, RoHS)

- Nuuk QA audit identifies a deviation from current WI that requires the WI to be corrected

- Production of a new model variant requires new or updated station instructions

All revisions are tracked in the WI Revision History section. The reason for every revision must be clearly stated (e.g., 'Issue found in PDI Inspection — added pull-force check for power cord connection'). Outdated WI versions are archived and not used on the production floor.

# 7. Records & Documentation

- Approved Work Instructions (all stations) — retained for the life of the product model + 3 years

- WI Revision History — maintained within each WI document

- WI Submission & Approval Log — maintained by Nuuk QA team

- Operator Training Register — maintained by partner; available for audit at all times

- CTQ Operator Competency List — shared between partner and Nuuk QA

- Process Audit Reports — retained for minimum 3 years

- Defect History Reports shared by Brand / Nuuk — retained and referenced in training material

- On-site Training Attendance Records (Nuuk-conducted) — retained for minimum 3 years

# 8. SOP Review & Version Control

This SOP shall be reviewed annually or upon any of the following triggers:

- Introduction of a new product model or assembly process

- Change in partner / Partner facility

- Significant training gap identified during process audit

- Regulatory update affecting assembly or safety testing requirements

- Request from Brand Representative.