STANDARD OPERATING PROCEDURE Incoming Quality Control (IQC) SKD Kit Inspection — Home Appliance Manufacturing

| Document No. | Revision | Effective Date | Review Date |
| --- | --- | --- | --- |
|  | Rev 00 |  |  |
| Department | Prepared By | Reviewed By | Approved By |
| Quality Assurance | Shivesh Thakral |  | Kartik Gupta |

# 1. Purpose

This Standard Operating Procedure (SOP) defines the process for Incoming Quality Control (IQC) of Semi-Knocked-Down (SKD) kits sourced from China for home appliance assembly. The objective is to ensure that all incoming materials conform to defined specifications before being released to the production floor, thereby preventing defective components from entering the manufacturing process.

# 2. Scope

This SOP applies to:

- All SKD kits received from approved manufacturers / partners.

- All packaging materials and accessories accompanying SKD kits.

- All personnel involved in receiving, inspection, and disposition of incoming materials.

- Quality Assurance (QA) and Production team at the Nuuk facility.

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| IQC Inspector (Indian manufacturer) | Perform sampling inspection of partner parts as per the approved Control Plan. Verify documents, conduct physical and packaging checks, update IQC records, and apply status labels. |
| QA Manager (Indian manufacturer) | Review inspection reports, make Accept / Conditional / Reject disposition decisions, and escalate findings to Nuuk QA Lead for final approval. |
| Nuuk QA Lead | Review IQC reports and issue final acceptance or rejection approval via email. If a major issue is identified that requires improvement in the next incoming lot, the Nuuk QA Lead shall highlight the issue to the China Partner team, record it in the Complaint Diary, and formally request a CAPA (Corrective and Preventive Action) if needed. |
| Production Manager (Indian manufacturer) | Confirm production readiness after Brand approval; escalate any production-floor quality issues traced to incoming materials. |

# 4. Reference Documents

- ISO 9001:2015 — Quality Management Systems

- Control Plan — Partner Parts (approved revision)

- Bill of Materials (BOM) from Manufacturer

- Purchase Order (PO) / Commercial Invoice

- Packing List from Manufacturer

# 5. Abbreviations

| Term | Full Form |
| --- | --- |
| IQC | Incoming Quality Control |
| SKD | Semi-Knocked-Down (partially assembled product kit) |
| BOM | Bill of Materials |
| CDF | Component Data Form / Constructional Data Form |
| NCR | Non-Conformance Report |
| CAPA | Corrective and Preventive Action |
| MRB | Material Review Board |
| AQL | Acceptable Quality Limit |
| CoC | Certificate of Conformance |
| PO | Purchase Order |

# 6. IQC Process Flow

The table below shows the complete IQC flow — who performs each step, who reviews / decides, and how issues are escalated.

| Step | Activity | Who Checks / Does | Who Reviews / Decides | If Issue Found |
| --- | --- | --- | --- | --- |
| 1 | Material receipt & GRN | Partner Store Team | Partner Store Team | Flag short/damaged shipment to IQC Inspector |
| 2 | Document verification (Invoice, BOM, CoC, Test Report) | IQC Inspector | QA Manager | Hold lot; raise document discrepancy to QA Manager |
| 3 | Physical & packaging inspection as per Control Plan | IQC Inspector | QA Manager | Raise NCR; QA Manager decides disposition (Accept / Hold / Reject) |
| 4 | IQC Report submission for Brand approval | QA Manager (Indian Partner) | Nuuk QA Lead | Brand Rep highlights issue to China Partner team; logs in Complaint Diary; requests CAPA |
| 5 | Final disposition — release to production or hold | IQC Inspector + Partner Store Team | Nuuk QA Lead (final approval via email) | Rejected lot moved to Hold Area; RTV / rework initiated |

# 7. Detailed Procedure

## 7.1  Material Receipt

Upon arrival of SKD kits from China:

- The Partner Store Team receives the consignment and cross-checks carton count against the Packing List and Commercial Invoice.

- External packaging is visually inspected for transit damage (crushed cartons, wet damage, open seals, etc.).

- A Goods Receipt Note (GRN) is raised with shipment details: PO number, supplier name, country of origin, lot / batch number, and quantity received.

- The consignment is placed in the IQC Inspection Bay / Quarantine Zone and tagged with a yellow 'UNDER INSPECTION' label pending IQC clearance.

## 7.2  Document Verification

Before any physical inspection, the IQC Inspector verifies the following documents supplied by the manufacturer:

| Document | Verification Point | Action if Missing / Deviant |
| --- | --- | --- |
| Commercial Invoice | SKU, quantity, unit value match PO | Raise document discrepancy; hold lot |
| Packing List | Carton count, part numbers, net / gross weight | Recount and reconcile; notify QA Manager |
| Bill of Materials (BOM) | Components match approved BOM revision | Hold and escalate to QA Manager |
| Manufacturer QC / Test Report | All parameters within spec; report signed by QA head | Request report; do not release without it |
| Certificate of Conformance (CoC) | Certifies material meets design / safety specification | Obtain from supplier before releasing lot |

All documents are archived (physical or scanned) with the IQC record for traceability.

## 7.3  Physical Inspection of SKD Materials

7.3.1  Sampling Plan

Sampling shall be performed as per the approved Control Plan for partner parts. Where the Control Plan does not specify a sample size, AQL 2.5 (major defects) and AQL 4.0 (minor defects) shall apply using ANSI/ASQ Z1.4 single sampling tables at General Inspection Level II.

| Lot Size (Units) | Sample Size | Max Allow. Major Defects | Max Allow. Minor Defects |
| --- | --- | --- | --- |
| 2 – 150 | 13 | 0 | 1 |
| 151 – 500 | 50 | 2 | 3 |
| 501 – 1200 | 80 | 3 | 5 |
| 1201 – 3200 | 125 | 5 | 7 |

7.3.2  Component Inspection Parameters

For each sampled SKD kit, the IQC Inspector shall evaluate the following:

| S.No. | Inspection Parameter | Method / Criteria | Defect Classification |
| --- | --- | --- | --- |
| 1 | Visual / Cosmetic Inspection | Check for scratches, dents, cracks, discoloration on all visible surfaces | Major (visible on finished product) / Minor (internal surfaces) |
| 2 | Component Completeness | Verify all parts listed in BOM are present per kit; count critical components | Critical (missing functional part) / Major (missing accessory) |
| 3 | Dimensional Check | Key dimensions measured with vernier calipers / gauges against drawing tolerance | Major (out of tolerance) |
| 4 | Label & Marking Verification | Check serial number, model number, rating label, safety markings (BIS / CE / RoHS) | Critical (missing mandatory mark) / Minor (misprint) |
| 5 | Electrical / Functional Spot-check | Basic electrical continuity / hi-pot test on sample units per product specification | Critical (fails safety test) |
| 6 | Material / Color Verification | Compare material grade and color to approved standard sample / CDF | Major (wrong material) / Minor (shade variation) |
| 7 | Hardware & Fasteners | Verify correct type, size, and quantity of screws, bolts, and brackets per BOM | Major |
| 8 | Packaging Deviation Check (Nuuk specific requirement) | Verify: (a) outer carton condition — no crushing, wetting, or open seals; (b) inner packaging (foam / bubble wrap / inserts) intact and component-protective; (c) carton markings correct — model no., batch no., qty, and country of origin match PO and packing list. | Major (impacts component protection or traceability) / Minor (cosmetic carton damage) |

## 7.4  Preparation & Submission of IQC Report

- On completion of inspection, the IQC Inspector compiles the IQC Inspection Report with all findings, photographs of defects, measurement data, and packaging deviation notes.

- The report is reviewed and signed by the QA Manager.

- The signed report (with supporting photos and test data) is shared with the Nuuk QA Lead via email within 24 hours of completing inspection.

- A copy is filed in the IQC Inspection Register (physical / digital) for audit traceability.

## 7.5  Brand Approval Process

The Nuuk QA Lead receives the IQC Inspection Report by email and reviews the findings:

- If there are minor deviations that are acceptable, the Nuuk QA Lead approves the lot with a documented concession via email, clearly stating the nature of the deviation and any conditions of use.

- If the lot is rejected, the Nuuk QA Lead communicates the rejection via email with reasons, and the material is moved to the Hold Area pending further action (see Section 7.6).

- No NC material shall be released to production without written email approval from the Nuuk QA Lead.

## 7.6  Non-Conformance & Hold Area Management

7.6.1  Rejected Lot Procedure

- Rejected material is immediately segregated and physically moved to the designated Hold Area.

- The Hold Area must be clearly demarcated with red boundary tape / signage: 'HOLD — DO NOT USE IN PRODUCTION.'

- A Non-Conformance Report (NCR) is raised and communicated to the supplier within 48 hours, with photographic evidence of all defects.

- The supplier must respond with a CAPA within 5 working days.

7.6.2  Disposition Options for Non-Conforming Material

- Return to Vendor (RTV): Full lot returned to supplier for replacement at supplier's cost to be handled by Nuuk Product team

- Sort & Rework: Supplier / local team performs 100% sorting; only conforming units released after re-inspection by IQC. If required, sorting and re-work to happen on production line

- Use-As-Is (Special Acceptance): Material accepted with documented Brand concession if defect does not impact function, safety, or regulatory compliance. Requires MRB approval

7.6.3  Re-Inspection After Rework

All reworked or replaced materials must undergo a full re-inspection by the IQC team before being released. Results are documented in a new IQC Inspection Report referencing the original NCR number.

# 8. Records & Documentation

- IQC Inspection Report — retained for minimum 3 years

- Non-Conformance Report (NCR) Log — retained for minimum 3 years

- IQC Inspection Register — master log of all lots inspected

- GRN (Goods Receipt Note) — linked to respective IQC report

- Complaint Diary — record of all major issues highlighted to China Partner team, including CAPA status

- Supplier CAPA Reports — retained for minimum 3 years

- Brand Approval Communications (emails) — archived per lot

# 9. SOP Review & Version Control

This SOP shall be reviewed annually or upon any of the following triggers

- Change in regulatory / certification requirements (BIS, CE, RoHS, etc.)

- Significant non-conformance trend identified in IQC data.

- Request from Brand for process update.