STANDARD OPERATING PROCEDURE Defect Analysis — Root Cause Analysis (RCA) & Corrective and Preventive Action (CAPA) Home Appliance Manufacturing — Assembly & Quality Operations

| Document No. | Revision | Effective Date | Review Date |
| --- | --- | --- | --- |
|  | Rev 00 |  |  |
| Department | Prepared By | Reviewed By | Approved By |
| Quality Assurance | Shivesh Thakral |  | Kartik Gupta |

NOTE: This SOP (SOP-DA-003) governs the defect escalation, root cause analysis (RCA), and corrective & preventive action (CAPA) process for all quality failures detected during Incoming Quality Control (IQC), In-Process Quality Control (IPQC), Outgoing Quality Check (OQC), and Pre-Delivery Inspection (PDI). It applies to defects originating from the China partner’s supplied components as well as from the Indian partner’s assembly operations.

# 1. Purpose

This Standard Operating Procedure (SOP) defines the process for systematic detection, escalation, root cause analysis (RCA), and corrective & preventive action (CAPA) management for all quality defects identified during the home appliance manufacturing and inspection lifecycle. It ensures that defects are not merely fixed but are permanently eliminated through structured problem-solving, validated corrective actions, and horizontal deployment across similar processes or products.

# 2. Scope

This SOP applies to:

- All quality defects detected during Incoming Quality Control (IQC) of SKD components received from the China partner

- Defects identified during In-Process Quality Control (IPQC) and Outgoing Quality Check (OQC) at the Indian partner’s assembly facility

- Defects detected during Pre-Delivery Inspection (PDI) before dispatch of finished goods

- The Brand Representative team responsible for escalation to the China partner and CAPA monitoring

- The Indian partner’s production and quality teams responsible for assembly-related CAPA

- The China partner’s team responsible for component-level CAPA

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Brand Representative | Escalate component-level issues to the China partner team. Review and accept or reject submitted CAPAs. Issue written communication upon CAPA acceptance. Monitor CAPA closure timelines. Maintain Complaint Diary. |
| Partner’s QC Team (Indian Partner) | Detects and records defects during IPQC and OQC. Escalate to the Brand Representative or partner’s production team. Validate corrective actions on the production line. Maintain OPL / training records. |
| Partner’s Production Team (Indian Partner) | Receive CAPA requests from Partner’s QC Team. Implement corrective and preventive actions on the assembly line. Participate in RCA activities. Provide OPL and training records as evidence & submit CAPA. |
| Nuuk QA Member(s) | Validate defect findings during IPQC / OQC / PDI. Co-review submitted 8D/CAPA. Confirm effectiveness of corrective actions during follow-up inspection (1–2 weeks post-implementation). Escalate unresolved issues to Nuuk QA Lead. |
| PDI Quality Team | Detects defects at PDI stage. Escalate findings to the Partner Process Quality Team / Nuuk Process QA . Ensure non-conforming units are quarantined before dispatch. |
| China Partner’s Team | Receive CAPA requests from Brand Representative for component-level defects. Submit 8D / CAPA in the prescribed format within the defined timeline. Implement corrective and preventive actions at the manufacturing source. |

# 4. Abbreviations

| Term | Full Form |
| --- | --- |
| RCA | Root Cause Analysis |
| CAPA | Corrective and Preventive Action |
| 8D | Eight Disciplines Problem Solving |
| IQC | Incoming Quality Control |
| IPQC | In-Process Quality Control |
| OQC | Outgoing Quality Check / Operational Quality Check |
| PDI | Pre-Delivery Inspection |
| NCR | Non-Conformance Report |
| OPL | One Point Lesson |
| 5-Why | Five-Why root cause analysis technique |
| SKD | Semi-Knocked-Down kit |
| WI | Work Instruction |
| D/A | Detection / Action |

# 5. Defect Detection & Escalation Pathways

The table below defines how defects detected at each quality gate are escalated and to whom the CAPA request is directed.

| Detection Stage | Defect Detected By | Escalation Path | CAPA Directed To |
| --- | --- | --- | --- |
| IQC (Incoming Quality Control) | Partner’s incoming inspector detects defects in incoming SKD components. | Partner QC Inspector → Partner QA Manager→ Nuuk QA Lead → China Partner’s Team | China Partner submits 8D/CAPA for component-level defect |
| IPQC (In-Process Quality Control) | Partner QC Inspector detects defect during assembly | Partner QC Inspector → Partner QC/Production Manager → Brand Representative (if escalation needed) | Partner’s QC Team requests CAPA from Partner’s Production Team |
| OQC (Outgoing Quality Check) | Partner QC / Nuuk QA detects functional defect on finished unit | Partner QC Inspector → Partner Production Manager/Nuuk Process QA → Brand Representative (for recurring / major issues) | Partner’s QC Team requests CAPA from Partner’s Production Team; Nuuk validates |
| PDI (Pre-Delivery Inspection) | PDI Quality Team detects defect before dispatch | PDI Nuuk Quality Team → Process Quality Team (Nuuk QA) → Partner Production/QA Manager | Process Quality Team (Nuuk) requests CAPA from Partner’s Production Team |

## 5.1  Detailed Escalation Rules

The following rules govern escalation decisions at each stage:

IQC — Component Defects (China Partner Origin):

- The Partner QA Inspector documents the defect in a Non-Conformance Report (NCR) and informs the Nuuk QA Lead immediately.

- The Nuuk QA team reviews the NCR and, if confirmed, notifies the China partner’s team and requests an 8D/CAPA submission within the applicable timeline.

- Non-conforming SKD lots are quarantined and not released for assembly until CAPA is reviewed and interim containment is confirmed.

IPQC / OQC — Assembly Process Defects (Indian Partner Origin):

- The Partner QC Inspector/Nuuk QA inspector raises an NCR and informs the Partner Production Manager.

- The Partner Production manager does the RCA & provides CAPA for critical issues using the prescribed 8D format.

- The Nuuk QA Member validates the defect finding and co-reviews the CAPA submission.

PDI — Pre-Delivery Defects:

- The PDI Quality Team immediately quarantines affected units and raises an NCR.

- The NCR is escalated to the Process Quality Team & Nuuk process QA.

- The Process Quality Team reviews the finding, determines root cause ownership (assembly process or component), and requests CAPA from the Partner’s Production Team accordingly.

- No quarantined units are dispatched until disposition is determined by the Nuuk QA Lead.

# 6. CAPA Submission Format — 8D Structure

The supplier / responsible party must submit the CAPA using the 8-Disciplines (8D) format. All 8 disciplines must be addressed in sequence. Partial submissions will not be accepted.

| 8D Discipline | Description & Requirements | Key Acceptance Criteria |
| --- | --- | --- |
| D1 — Define the Problem | Clear, specific problem statement: What is the defect? Where was it detected? How many units are affected? What is the severity (Critical / Major / Minor)? | NCR Number, Product Model, Date, Detection Stage |
| D2 — Define the Team | Name all team members involved in the RCA and CAPA activity, including their roles (e.g., QA Engineer, Production Supervisor, Process Engineer). | Team Lead designated; cross-functional representation required |
| D3 — Interim Containment Action (ICA) | Immediate actions taken to prevent defective products from reaching the next stage or the customer. Include: segregation of affected lots, 100% re-inspection, hold tags applied. | ICA implemented within 24 hours of NCR issuance |
| D4 — Root Cause Analysis (RCA) | Two-track RCA required: - Detection RCA: Why was the defect not detected earlier? (process gap, inspection gap) - Occurrence RCA: Why did the defect occur? (process, material, human, machine, method, environment) Mandatory tools: - 5-Why analysis (for both detection and occurrence) - Fishbone (Ishikawa) Diagram where the root cause is multi-factorial or unclear | Both detection and occurrence root causes must be identified separately |
| D5 — Corrective Action on Detection | Actions to improve the detection system so the defect is caught earlier / reliably in future. E.g., adding an inspection step, updating inspection criteria, modifying test method. | Linked directly to detection RCA findings |
| D6 — Corrective Action on Occurrence | Actions to eliminate the root cause of defect occurrence. E.g., process parameter change, WI revision, tooling correction, component specification change. | Linked directly to occurrence RCA findings |
| D7 — Validation of Action Plans | Evidence that corrective actions were implemented and are effective. Conducted 1–2 weeks after implementation. Includes: re-inspection results, process monitoring data, confirmation that defect has not recurred. | Validation report submitted to customer (Brand Rep / Nuuk QA) for review |
| D8 — Horizontal Deployment | Apply learnings to similar products, processes, or assembly lines where the same defect could potentially occur. Update relevant Work Instructions, Control Plans, and training materials. | Horizontal deployment list and evidence of updates submitted |
| Closure of Problem Statement | Formal closure of the CAPA once all 8D steps are validated, horizontal deployment is complete, and no recurrence has been observed during the monitoring period. | Written closure communicated by customer (Brand Rep) via email |

## 6.1  Supporting Documents Required with CAPA Submission

- Completed 8D CAPA form (in the format defined above or supplier’s equivalent format — subject to customer approval)

- 5-Why analysis worksheet (separate sheets for Detection RCA and Occurrence RCA)

- Fishbone (Ishikawa) Diagram — mandatory when root cause is multi-factorial or unclear; recommended in all cases

- One Point Lesson (OPL) record or Training Record confirming operators have been trained on the revised process / WI

- Updated Work Instruction (WI) or Control Plan excerpt (where applicable)

- Validation data / re-inspection report (submitted as part of D7 — must be provided before CAPA can be closed)

- Horizontal deployment evidence (list of processes/products covered, updated documents)

# 7. CAPA Review, Acceptance & Closure

## 7.1  Review Process

Upon receipt of the CAPA submission, the following review sequence applies:

- Nuuk QA Lead  reviews the submission against the 8D checklist within 5 working days of receipt.

- If the submission is incomplete or the root cause is not adequately justified, it is returned to the supplier / production team with written comments specifying the gap. The supplier must resubmit within the timeline specified in the return communication.

- If the submission is satisfactory, the reviewer communicates acceptance via email, confirming each 8D step has been reviewed and approved.

- Validation evidence (D7) is reviewed separately once submitted (1–2 weeks post-implementation). Acceptance of the 8D form does not constitute CAPA closure.

- Final CAPA closure is communicated in writing (email) by the Nuuk QA Lead once D7 validation is accepted and D8 horizontal deployment is confirmed.

## 7.2  CAPA Timelines

CAPA timelines must comply with applicable electrical appliance quality standards. The indicative timelines below apply unless a specific standard mandates otherwise:

| CAPA Milestone | Critical Defect | Major Defect | Minor Defect |
| --- | --- | --- | --- |
| Interim Containment Action (D3) | Within 24 hours | Within 48 hours | Within 72 hours |
| Full 8D Submission (D1–D6) | Within 7 working days | Within 10 working days | Within 15 working days |
| Validation Evidence (D7) | 1–2 weeks post-implementation | 1–2 weeks post-implementation | 2–3 weeks post-implementation |
| Horizontal Deployment (D8) & Closure | Within 30 days | Within 45 days | Within 60 days |

Note: These timelines are indicative. Timelines defined within the accepted 8D/CAPA document itself are binding. Failure to meet defined timelines must be escalated to the Nuuk QA Lead.

## 7.3  CAPA Acceptance Communication

- Customer review and acceptance is communicated via email.

- The acceptance email must reference the NCR number, CAPA reference, product model, and the specific 8D step(s) being accepted.

- Partial acceptance (e.g., D1–D6 accepted, D7 pending) must be clearly stated in the communication.

- The supplier must retain copies of all customer acceptance communications as part of the quality record.

# 8. RCA Tools — Guidance

## 8.1  5-Why Analysis

The 5-Why technique involves repeatedly asking “Why?” to drill down from a symptom to the root cause. Two separate 5-Why chains must be conducted:

- 5-Why for Occurrence: Why did the defect happen?

- 5-Why for Detection: Why was the defect not caught before reaching the next stage?

Each “Why” answer must be supported by evidence (process data, inspection records, WI review). The analysis should continue until a controllable, actionable root cause is reached — not just a human error statement.

## 8.2  Fishbone (Ishikawa) Diagram

The fishbone diagram is mandatory when the root cause analysis involves multiple contributing factors, or when the 5-Why chain alone does not clearly identify a single root cause. The fishbone should cover the standard 6M categories:

- Man — Operator skill, training gap, fatigue

- Machine — Equipment condition, calibration, tooling

- Method — Process parameter, work instruction accuracy

- Material — Component quality, specification compliance

- Measurement — Inspection method, gauge accuracy, sampling plan

- Environment — Temperature, humidity, ESD, housekeeping

## 8.3  One Point Lesson (OPL)

An OPL is a single-page visual training document summarising the key learning from a defect event. It must be created whenever:

- A new or recurring defect type is identified

- A corrective action requires a change to operator behaviour or method

- A Work Instruction has been revised as a result of the CAPA

The OPL must be reviewed and signed by the relevant operators and their supervisor. The signed OPL record is submitted as part of the CAPA closure evidence.

# 9. Defect Classification & NCR Raising

All defects must be classified before an NCR is raised. The classification drives escalation urgency and CAPA timelines.

| Classification | Definition | Examples |
| --- | --- | --- |
| Critical | Defect that renders the product unsafe, non-functional, or non-compliant with regulatory / safety standards. | Electrical short, missing safety label, failed hi-pot test, fire/shock hazard |
| Major | Defect that significantly impairs product functionality or appearance beyond customer acceptance limits. | Motor failure, display malfunction, significant cosmetic damage, missing component |
| Minor | Defect that has minimal impact on function or appearance; product still meets basic customer requirements. | Minor surface scratch within tolerance, slight colour variation, label misalignment within spec |

The NCR must document: NCR number, date, detection stage, product model, batch/lot reference, defect description, quantity affected, classification, and the inspector’s name.

# 10. Records & Documentation

- Non-Conformance Report (NCR) Log — retained for minimum 3 years

- 8D / CAPA Submission (supplier format) — retained per batch / lot

- 5-Why Analysis Worksheets — retained with CAPA record

- Fishbone Diagram — retained with CAPA record (where applicable)

- OPL / Training Records — retained per training event

- CAPA Validation / Re-inspection Reports — retained per CAPA

- Customer Acceptance Emails — archived per CAPA reference

- Complaint Diary — running log of all escalated issues and their CAPA status

- Horizontal Deployment Evidence (updated WIs, Control Plans, operator sign-offs) — retained per CAPA

# 11. SOP Review & Version Control

This SOP shall be reviewed annually or upon any of the following triggers:

- Change in product model, assembly process, or component supplier

- Recurring CAPA failure or non-closure within defined timelines

- Change in applicable regulatory / quality standard requirements

- Request from Brand Representative for process update